# 7* Low Level Design  >1500 words

This section provides low-level design descriptions that directly support construction of modules. Normally this section may be split into separate documents for different areas of the design.  For each component we now need to break it down into its fundamental units or modules. Each module or block may be hardware or software or a subsystem implemented using hardware and software. Make sure to provide a well-commented schematic of all modules and/or system blocks described in the system block diagram (SBD).

Low-level design is a detailed description of every module of the system. It describes every module in detail by incorporating the logic behind every component in the system. It delves deep into every specification of every system, providing a micro-level design.

Low-level designs are created and implemented by designers and developers. This design breaks down high-level solutions into the smallest details.

Must include hardware circuit schematics and program flowchart for each module.

Example of circuit schematics: http://www.arduino.cc/en/uploads/Main/arduino-uno-schematic.pdf

Example of flowcharts of your programs: http://www.mdpi.com/electronics/electronics-03-00636/article_deploy/html/images/electronics-03-00636-g011-1024.png

First give a short introduction and define the modules/blocks

## *Module i (for i = 1 to n) (change the italics to the name of your module)

Provide or reference a detailed description, schematic and/or diagrams of this module. Repeat this section for each module i.

State clearly who is responsible for which module/task

### Processing narrative for module i (change italics to the name of your module)

A processing narrative for module i is presented.

### Module i interface description (change italics to the name of your module)

A detailed description of the input and output interfaces of the module with other modules in the system, with other software or systems or module is presented.

### Module i processing details (change italics to the name of your module)

A detailed description for each module is presented, including:

Hardware,

Algorithm,

Local data structures,

Design constraints, limitations, performance issues, etc.

End of the template

More Explanations

## What to Include in Low-Level Design:

### 1. Detailed Circuit Designs

- Complete schematics for each electrical subsystem (e.g., motor control, power regulation, sensor circuits).

- Component values (resistors, capacitors, etc.), part numbers, and justification for selection.

- Simulation results, if applicable (e.g., from LTspice, Multisim).

### 2. PCB Design (if applicable)

- Layout screenshots.

- Layer stack-up details.

- Trace width, spacing rules, and via usage.

- Justification of component placement and routing decisions.

### 3. Firmware/Software Details

- Function breakdown: Describe each function/subroutine and its purpose.

- Code structure: File/module organization, data structures, key algorithms (e.g., PID controller).

- I/O pin mapping: Microcontroller pin assignments to peripherals (GPIO, PWM, ADC, etc.).

- Interrupts and Timers: Usage and configuration.

### 4. Communication Protocols

- Details of protocols used (e.g., I²C, SPI, UART, CAN).

- Addressing, timing, and data formats.

- Packet structure if you designed your own protocol.

### 5. Sensor and Actuator Interfaces

- Voltage levels, pull-up/down requirements, calibration methods.

- Analog vs digital interface details.

### 6. Mechanical Interface Control

- Low-level signal handling for actuators (e.g., PWM signal details for servo motors).

- Limit switch integration or encoder feedback handling.

### 7. Timing Diagrams or State Machines

- FSMs with state transitions, inputs, and outputs.

- Timing diagrams for signal sequences (e.g., sensor reads, PWM cycles).

### 8. Power Supply Details

- Voltage regulation stages.

- Current draw estimates per subsystem.

- Heat dissipation considerations (if any).

### 9. Safety and Fault Handling

- Overcurrent protection logic.

- Emergency stop logic.

- Software/hardware failsafe.

### 10. Testing Interfaces or Debug Tools

- UART/USB outputs for logging.

- Onboard debugging features (e.g., LEDs, test pins, serial consoles).

## Low-Level Design: Motor Control Subsystem

### 1. Overview

The motor control subsystem is responsible for rotating the robot base, moving the robotic arm, and actuating the gripper. It uses three SG90 servo motors, each controlled by a PWM signal generated by an STM32 microcontroller.

### 2. Circuit Schematic

- Each SG90 servo is powered by a separate 5V supply rail with a 220 µF decoupling capacitor close to the servo pin.

- PWM signals are generated from STM32 pins PA8, PA9, and PA10.

- A 1kΩ resistor is placed in series with the signal line to reduce ringing.

- All ground lines (STM32, servos, power supply) are connected to a common ground.

(Attach LTspice or KiCad schematic image here.)

### 3. Component List

| Component | Part Number | Quantity | Notes |
| --- | --- | --- | --- |
| Servo Motor | SG90 | 3 | Base, Arm, Gripper |
| Microcontroller | STM32F103C8T6 | 1 | "Blue Pill" board |
| Capacitor | 220 µF | 3 | Per servo, for voltage stability |
| Resistor | 1 kΩ | 3 | Signal line protection |

### 4. Firmware Implementation

#### PWM Configuration (STM32 using HAL library):

c

CopyEdit

// Timer configuration (TIM1)

htim1.Instance = TIM1;

htim1.Init.Prescaler = 71;

htim1.Init.CounterMode = TIM_COUNTERMODE_UP;

htim1.Init.Period = 19999; // For 50 Hz PWM (20ms)

HAL_TIM_PWM_Init(&htim1);

// Duty cycle for servo (0° = 500us, 180° = 2500us)

__HAL_TIM_SET_COMPARE(&htim1, TIM_CHANNEL_1, 1500); // Midpoint (90°)

### 5. Pin Mapping

| Function | STM32 Pin | Servo Label |
| --- | --- | --- |
| Base Rotation | PA8 | Servo 1 |
| Arm Movement | PA9 | Servo 2 |
| Gripper Actuator | PA10 | Servo 3 |

### 6. Safety Features

- Software includes position limit checks to avoid over-rotation.

- Current draw is monitored via external multimeter during testing.

- Manual emergency stop disables PWM output.

### 7. Testing Procedure

- PWM range was calibrated by varying duty cycles and visually confirming servo angles.

- Oscilloscope used to verify PWM pulse width (500 µs – 2500 µs).

- Stress-tested each servo under load for 5 minutes continuously.